# SUMMARY OF CHANGES

WOBBLE MODE:

The original sweep mode of stepping up and down between two frequencies has been

changed to what is more commonly known as a “wobble”. This function now operates

from keys 4, 5 and 6.

KEY-4		Change wobble direction.

KEY-5		Start / Halt / Resume slow wobble.

KEY_6	Fast wobble.

## SWEEP MODE:

Slow / Fast sweeping is now a flyback sweep, stepping from the start frequency to the finish frequency, at which time a flyback to the start frequency occurs. The sweep is then continuously repeated until halted. The start frequency is loaded when halted.

A single sweep mode has been implimented using KEY-7. Once the sweep has been halted, pressing this key produces a sweep from the start frequency to the finish

frequency . When the finish frequency is reached, a flyback to the start frequency

occurs and the sweep is then halted again. This is repeated each time KEY-7 is pressed.

A positive going trigger pulse is generated on PORTA,6 (PIC pin 15) evey time the start frequency is sent to the DDS.

KEY-7		Single Sweep.

KEY-8		Start / Halt / Restart sweep.

KEY-9		Fast Sweep.

A mode display on line 1 of the LCD has been added.

SOURCE CODE NOW NAMED SWEEPGEN+

Version Beta_0

The following sections of the source code have been modified.

FLAG DEFINITIONS

TEXT STRING TABLE

SWEEP FUCTION ROUTINE

KEY FUNCTION ROUTINE

ADDED DISP_MODE ROUTINE

COPY ROUTINE (ARG4_TO_ARG2) ADDED

CONSTANT DEFINITION TABLES